YO

YOLO HAND-GESTURE CONTROLLED DUSTBIN

Technical Documentation for Computer Vision

Instructor: Dr. Tarek

Academic year: 2025-2026

COMPUTER

ENGINEERING

Submitted by:

Amr Edris, 22-101097

Mourad El-Shorbagy, 22-101026

Table of Contents

1. Project Overview	4

Target Audience & Versatility	4

Command Logic	4

Advanced Real-Time Image Processing	4

The ESP-32 Integration	4

2. Initialization	5

3. Dataset and Data Preparation	6

3.1 Setting file paths	6

3.2 Extract all classes that aren’t fist or palm in a list	6

3.3 Set data paths for fist and palm	7

3.4 Understanding the JSON file	7

3.5 Cleaning the Data from excess	10

4. Transforming to data frame for yolo	11

4.1 Data Consolidation and Unified Formatting	12

4.2 Dataset Stratification and Distribution	13

5. YOLO Setup	14

5.1 Yolo directory infrastructure setup	14

5.2 dataset_yaml format	15

5.3 Format YOLO’s data	17

5.5 Model Training Configuration	21

5.6 Results of Training	22

6. Testing	23

7. Preprocessing	24

8. Real-Time Inference and Interaction Loop	25

Core Implementation Logic	25

Drawing Bounding Boxes for feedback	26

9. Bonus: Hardware Interface to Robot	28

9.1 Laptop Code	28

9.2: Microcontroller Code	30

# 1. Project Overview

The Hand-Gestured Dustbin is an integrated computer vision and robotics project designed to interpret human intent through real-time hand gestures. The system's core intelligence is built upon the YOLOv8 (You Only Look Once) object detection architecture. To facilitate high-accuracy recognition, the model was trained using the HaGRID (Hand Gesture Recognition Image Dataset) 30k 384p sample.

## Target Audience & Versatility

The primary objective of this project is to assist individuals with limited mobility. While a dust bin serves as the initial prototype, the underlying framework is highly versatile. The same gesture-based control logic can be adapted for far more purposes rather than this one.

## Command Logic

The system is trained to classify gestures into three primary functional states:

- Fist (Command: Approach): Signals the device to navigate toward the user’s current position.

- Palm (Command: Reset): Signals the device to return to its designated initial or "home" position.

- Others (Background Filtering): Includes a wide range of gestures (e.g., peace signs, "OK" signs, or random hand movements) used to teach the model what to ignore. This minimizes false positives and ensures high operational reliability.

## Advanced Real-Time Image Processing

To enhance detection accuracy in cluttered environments, the project implements a two-pass "focus" pipeline":

- Localization Pass: The model identifies the hand's bounding box.

- Gaussian Blurring: The system applies a 41x41 kernel blur to the background.

- Classification Pass: The model re-scans the sharp hand region against the blurred background, capturing the gesture with significantly higher clarity.

## The ESP-32 Integration

The ESP-32 will receive the signal over the wifi in a UDP packet, and based on the signal it will control the movement of the dust-bin.

# 2. Initialization

import os

import json

import pandas as pd

import cv2

import matplotlib.pyplot as plt

from tqdm import tqdm # For progress bar

from sklearn.model_selection import train_test_split

import shutil

from ultralytics import YOLO

import numpy as np

ultralytics (YOLO): Provides the framework for training the YOLOv8n model and performing real-time inference to detect hand gestures.

cv2 (OpenCV): Handles camera feed integration, frame capture, and image preprocessing, specifically the implementation of Gaussian blurring for focusing on the hand.

pandas: Facilitates the management of structured data by organizing cleaned annotations and file paths into DataFrames for model ingestion.

json: Enables the extraction and parsing of annotation data from the original HaGRID dataset files.

sklearn (train_test_split): Used for the statistical division of the dataset into training, validation, and testing sets while maintaining class distribution via stratification.

os & shutil: Manage file system interactions, including directory creation and the physical transfer of image files into the YOLO folder structure.

numpy: Executes numerical operations required for coordinate transformations and processing model output arrays into a format compatible with OpenCV.

tqdm: Implements visual progress bars to monitor the status of long-running data cleaning and formatting tasks.

matplotlib.pyplot: Utilized for the visualization of data samples and training performance metrics within the development environment.

# 3. Dataset and Data Preparation

## 3.1 Setting file paths

Annotation_path = "./hagrid-sample-30k-384p/ann_train_val"

Images_path = "./hagrid-sample-30k-384p/hagrid_30k"

if os.path.exists(Annotation_path) and os.path.exists(Images_path):

print("Paths exist.")

else:

print("Path doesn't exist")

We set our annotation path and images path so we don’t have to write the same paths redundantly.

## 3.2 Extract all classes that aren’t fist or palm in a list

all_classes = [] # To store all class names

if os.path.exists(Annotation_path): # Check if annotation path exists

for filename in os.listdir(Annotation_path): # List all files in annotation directory

if filename.endswith('.json'): # Process only JSON files

all_classes.append(filename.split('.')[0]) # Get name without .json for example call.json -> call

else:

print(f"ERROR: Annotation path not found: {Annotation_path}")

# Handle error appropriately, maybe exit()

# Filter out 'fist' and 'palm'

other_classes = [cls for cls in all_classes if cls not in ['fist', 'palm']] # List comprehension to filter classes that aren't 'fist' or 'palm'

print(f"Found {len(other_classes)} other classes to process:") # Print number of other classes found

print(other_classes) # Print the list of other classes

Extracted all other classes names in a list.

## 3.3 Set data paths for fist and palm

fist_ann = os.path.join(Annotation_path, "fist.json") # Path to fist annotation file

palm_ann = os.path.join(Annotation_path, "palm.json") # Path to palm annotation file

if os.path.exists(fist_ann) and os.path.exists(palm_ann): # Check if both paths exist

print("Paths exist.")

fist_images = os.path.join(Images_path, "train_val_fist") # Path to fist images directory

palm_images = os.path.join(Images_path, "train_val_palm") # Path to palm images directory

if os.path.exists(fist_images) and os.path.exists(palm_images): # Check if both paths exist

print("Paths exist.")

Made paths for fist and palms annotation and images so we don’t have to write the same paths redundantly.

## 3.4 Understanding the JSON file

if os.path.exists(fist_ann):

# File exists, now attempt to open and load

with open(fist_ann, 'r') as f:

# Note: This line can still crash if fist.json is invalid!

fist_data = json.load(f)

print(f"Successfully opened and attempted to load {fist_ann}")

else:

print(f"Error: The file '{fist_ann}' was not found.")

if os.path.exists(palm_ann):

# File exists, now attempt to open and load

with open(palm_ann, 'r') as f:

# Note: This line can still crash if palm.json is invalid!

palm_data = json.load(f)

print(f"Successfully opened and attempted to load {palm_ann}")

else:

print(f"Error: The file '{palm_ann}' was not found.")

all_other_data = {} # Dictionary to store data for other classes

for class_name in other_classes:

class_json = os.path.join(Annotation_path, f"{class_name}.json") # Path to class JSON file

if os.path.exists(class_json):  # Check if file exists

with open(class_json, 'r') as f:

class_data = json.load(f) # Load JSON data

all_other_data[class_name] = class_data # Store in dictionary

print(f"Successfully opened and attempted to load {class_json}")

else:

print(f"Error: The file '{class_json}' was not found.")

Fist and Palm Classification

Annotations for the primary control gestures, fist and palm, are handled individually to ensure rapid access during the training pipeline:

- Validation Check: The system first verifies the physical existence of the JSON files (fist_ann and palm_ann) using the os.path.exists method.

- Independent Storage: Upon successful verification, the data is parsed via json.load() and stored into dedicated variables: fist_data and palm_data.

- Structure: Each variable contains a dictionary where the keys are unique image identifiers and the values are nested dictionaries containing bounding box coordinates and metadata.

Secondary Class Aggregation (Others)

To manage the remaining 16 gesture classes, the system employs an automated loop to consolidate data into a single multi-dimensional structure:

- Dynamic Mapping: The system iterates through the other_classes list, constructing file paths dynamically for each corresponding JSON file.

- Nested Dictionary Format: All non-primary gesture data is stored within a master dictionary named all_other_data.

- Key-Value Relationship:

  - Key: The specific class name (e.g., "call", "mute", "ok").

  - Value: The complete dictionary of annotations associated with that specific gesture class.

- count = 0

- print("First 5 items:")

- for key, value in fist_data.items():

- if count < 5:

- print(f"  Key: {key}")

- print(f"  Value: {value}\n") # Add a newline for readability

- count += 1

- else:

- break # Stop the loop after 5 items

This code snippet is supposed to print the first 5 items of the JSON, I will display the first one for reference:

Key: 003a238e-6ea2-48b0-ad4c-462f9dc655df

Value: {'bboxes': [[0.62957251, 0.39665797, 0.20516206, 0.16012795]], 'labels': ['fist'], 'leading_hand': 'right', 'leading_conf': 0.8, 'user_id': 'b79fa61074d12fdac1e695233d33143715fa505c2bf6b90edd40a75107e89233'}

Key: Unique identifier of the image (image id)

Values:

1. bboxes: Normalized coordinates for the bounding box ([top-left-X-position, top-left-Y-position, width, height]).

2. labels: The object class name detected.

3. leading_hand: label identifying the active hand in the image.

4. leading_conf: the percentage confidence that the leading hand is the correct hand

5. user_id: user id of the person who contributed with the image

We printed the JSON for all to test if everything was successful.

## 3.5 Cleaning the Data from excess

For fist data:

cleaned_fist_data = {} # Dictionary to hold cleaned fist data

for image_id, annotations in tqdm(fist_data.items(), desc="Cleaning Fist Data"): # Iterate over each image in fist data with progress bar

image_path = os.path.join(fist_images, f"{image_id}.jpg") # Construct full image path

# Check if the image file exists

if os.path.exists(image_path):

# If it exists, add it to the clean dictionary

cleaned_fist_data[image_id] = annotations # Store valid entry

The data cleaning process ensures that the model is trained only on valid pairs of annotations and image files. This is critical because the project utilizes a sample subset of the larger HaGRID dataset, meaning many entries in the original JSON files do not have corresponding images in the local storage.

Validation and Verification Process

- Initialization: A new dictionary, cleaned_fist_data, is initialized to store only the entries that pass the verification check.

- Iteration: The system iterates through every image identifier and its associated annotations within the fist_data structure.

- Progress Tracking: The tqdm library is utilized to provide a real-time progress bar during the iteration of the thousands of potential entries.

- Path Construction: For each entry, the system constructs a full file path by combining the fist_images directory path with the image_id and the .jpg extension.

- Existence Check: The os.path.exists() function is called to verify if the physical image file is present on the disk at the constructed path.

- Data Retention: If the image exists, the image_id and its corresponding annotations are copied into the cleaned_fist_data dictionary; otherwise, the entry is discarded.

This applies to palm and other classes as well

Results:

Original fist entries: 27764

Clean fist entries: 1735 (Removed 26029)

Original palm entries: 28326

Clean palm entries: 1770 (Removed 26556)

Original others entries checked: 453233

Clean others entries found: 28328 (Removed 424905)

TOTAL CLEANED ENTRIES:

31833

# 4. Transforming to data frame for yolo

The use of a DataFrame is essential for standardizing disparate data sources into a single, structured format compatible with machine learning workflows. It enables:

- Data Consolidation: Merging the separate fist, palm, and others dictionaries into a single table for unified processing.

- Label Mapping: Efficiently converting string names (e.g., "fist") into the integer IDs (0, 1, 2) required by the YOLO architecture.

- Stratified Splitting: Utilizing train_test_split to ensure the 80/10/10 ratio of training, validation, and testing data maintains a balanced distribution of all gesture classes.

- Coordinate Calculation: Performing batch calculations to transform HaGRID coordinates (xmin,ymin) into the YOLO-required format (xcenter,ycenter).

## 4.1 Data Consolidation and Unified Formatting

The final stage of data preparation involves merging localized dictionaries into a single, global list structure to facilitate the creation of the training DataFrame.

Core Logic Implementation

The system processes each category (fist, palm, and others) by iterating through their respective cleaned dictionaries and appending them to a master list named all_data:

for image_id, annotations in cleaned_fist_data.items():

annotations['image_id'] = image_id

annotations['class'] = 'fist'

annotations['target_class'] = 'fist'

all_data.append(annotations)

This action is done to all gestures data.

Key Technical Actions

- Standardization: Every image entry is updated with three new attributes: image_id for identification, class for original labeling, and target_class for model training.

- Target Class Mapping: While fist and palm maintain their specific names, all sixteen secondary gestures are assigned to the others target class.

- DataFrame Conversion: The completed all_data list is passed to the Pandas DataFrame() constructor. This transforms the raw dictionary data into a structured table format, which is essential for the next steps of data splitting and YOLO label generation.

- final_clean_df = pd.DataFrame(all_data)

Result:

## 4.2 Dataset Stratification and Distribution

Partitioning the DataFrame into three distinct subsets required for machine learning: Training, Validation, and Testing.

Two-Stage Splitting Process

The system uses a two-stage approach to achieve a precise 80/10/10 ratio:

1. Initial Split: 80% of the data is allocated to the Training set, while the remaining 20% is held in a temporary variable (temp_df).

- # 1. First split: Separate 80% for training and 20% for 'temporary' (val + test)

- train_df, temp_df = train_test_split(

- final_clean_df,

- test_size=0.20,

- random_state=42,

- stratify=final_clean_df['class']

- )

2. Secondary Split: The 20% temporary data is divided equally (50/50), resulting in a final distribution where Validation and Testing each receive 10% of the original total.

- # 2. Second split: Separate the 20% 'temp' into 50% validation and 50% test

- # (This results in 10% of the total for each)

- val_df, test_df = train_test_split(

- temp_df,

- test_size=0.50,

- random_state=42,

- stratify=temp_df['class']

- )

Key Implementation Details

- Class Mapping: A class_map dictionary is defined to assign unique integer IDs to the gesture categories: fist (0), palm (1), and others (2).

- class_map = {

- 'fist': 0,

- 'palm': 1,

- 'others': 2

- }

- Stratification: The stratify parameter is applied to the class column. This ensures that each subset contains a balanced proportion of every gesture type, preventing the model from becoming biased toward any specific class.

- Randomness: A random_state of 42 is used to ensure that the data split remains consistent across different execution runs.

# 5. YOLO Setup

## 5.1 Yolo directory infrastructure setup

To train a YOLO model, the dataset must be organized into a specific hierarchical folder structure. This code automates the creation of that infrastructure to ensure the training script can locate images and their corresponding labels correctly.

# Define the root directory for your YOLO dataset (e.g., in your project folder)

# MAKE SURE 'yolo_dataset' folder DOES NOT already exist where you run this,

# or choose a different name.

yolo_dir = './yolo_dataset' # Use './' for current directory

# Define subdirectories

images_train_dir = os.path.join(yolo_dir, 'images', 'train')

images_val_dir = os.path.join(yolo_dir, 'images', 'val')

images_test_dir = os.path.join(yolo_dir, 'images', 'test')

labels_train_dir = os.path.join(yolo_dir, 'labels', 'train')

labels_val_dir = os.path.join(yolo_dir, 'labels', 'val')

labels_test_dir = os.path.join(yolo_dir, 'labels', 'test')

# Create the directories

os.makedirs(images_train_dir, exist_ok=True)

os.makedirs(images_val_dir, exist_ok=True)

os.makedirs(images_test_dir, exist_ok=True)

os.makedirs(labels_train_dir, exist_ok=True)

os.makedirs(labels_val_dir, exist_ok=True)

os.makedirs(labels_test_dir, exist_ok=True)

print(f"Created YOLO folder structure inside: {yolo_dir}")

At start, it establishes a root directory as yolo_dataset and fills it with two main branches: images and labels. Within each branch, three subdirectories are created to match the previously defined data splits:

- Train: Contains data used to teach the model.

- Val (Validation): Contains data used to fine-tune the model during training.

- Test: Contains data used to evaluate the final performance of the model.

## 5.2 dataset_yaml format

Core Functions of the YAML File:

- Path Specification: It defines the root directory and the specific paths to the train, val, and test image folders created during the directory setup.

- Class Identification: It explicitly lists the total number of classes (nc) the model is expected to detect.

- Label Mapping: It provides an ordered list of class names (e.g., 0: fist, 1: palm, 2: others) that match the integer IDs written in the label text files.

When the model.train() function is called, it references the YAML file (data=yaml_path) to locate the images and verify that the labels match the expected output categories. Without this file, the YOLO framework cannot interpret the raw folders and .txt labels generated by the format_yolo_data function.

import yaml

# Define the configuration based on your paths and 3 classes

dataset_yaml = {

'path': '/content/projectcv/yolo_dataset/', # The root path we established

'train': 'images/train',                    # Relative to root path

'val': 'images/val',                        # Relative to root path

'test': 'images/test',                      # Relative to root path

'nc': 3,                                    # Number of classes

'names': ['fist', 'palm', 'others']         # Class names in order (0, 1, 2)

}

# Define where to save the file

yaml_path = '/content/projectcv/yolo_dataset/dataset.yaml'

# Create the directory if it doesn't exist yet

os.makedirs(os.path.dirname(yaml_path), exist_ok=True)

# Write the dictionary to the YAML file

with open(yaml_path, 'w') as f:

yaml.dump(dataset_yaml, f, default_flow_style=False, sort_keys=False)

print(f"Successfully created {yaml_path}")

print("--- YAML Content ---")

with open(yaml_path, 'r') as f:

print(f.read())

Format of the dataset yaml:

path: ./yolo_dataset/       # Specifies the root directory containing the entire dataset

train: images/train         # Defines the path to the training images relative to the root

val: images/val             # Defines the path to the validation images relative to the root

test: images/test           # Defines the path to the testing images relative to the root

nc: 3                       # Number of Classes: Defines the total count of gesture categories

names: ['fist', 'palm', 'others']  # Maps the integer IDs (0, 1, 2) to their human-readable labels.

## 5.3 Format YOLO’s data

We need to store the images in the yolo folders and turn annotations to yolo form using txt

This is a function that turns the data frame to format its data.

def format_yolo_data(df, split_name):

takes in the dataframe and the split name and formats their data in the following way:

1. Path construction and iteration

image_dir = os.path.join(yolo_dir, 'images', split_name)

label_dir = os.path.join(yolo_dir, 'labels', split_name)

# Loop through each row in the DataFrame

for _, row in tqdm(df.iterrows(), total=len(df), desc=f"Formatting {split_name}"): # Use tqdm for progress bar

image_dir: Establishes the destination folder for physical image files.

label_dir: Establishes the destination folder for the corresponding YOLO text labels.

The processing loop is designed for both efficiency and user feedback:

- df.iterrows(): Iterates through each entry in the DataFrame, returning the row index (discarded as _) and the row data (row).

- tqdm(...): Wraps the iterator to provide a real-time visual progress bar.

- total=len(df): Informs the progress bar of the total number of images to be processed for accurate completion estimates.

- desc=f"Formatting {split_name}": Displays a specific text label next to the bar to identify which dataset split is currently being generated.

2. Collect the variables in the row:

image_id = row['image_id'] # Image identifier

class_name = row['target_class'] # Target class (fist, palm, others)

org_class_name = row['class'] # Original class name

bboxes = row['bboxes'] # List of bounding boxes for this image

3. Copy image file from HaGRID source to YOLO destination folder

# Construct the SOURCE image path

source_img_folder = os.path.join(Images_path, f"train_val_{org_class_name}") # Source folder based on original class

source_img_path = os.path.join(source_img_folder, f"{image_id}.jpg") # Full source image path

# Construct the DESTINATION paths

dest_img_path = os.path.join(image_dir, f"{image_id}.jpg") # Destination image path

dest_label_path = os.path.join(label_dir, f"{image_id}.txt") # Destination label path

#  Copy the image file

if os.path.exists(source_img_path): # Check if source image exists

shutil.copyfile(source_img_path, dest_img_path) # Copy to destination

Path Construction

- Source Folder Identification: The system constructs the source_img_folder path by appending the original gesture class name (e.g., train_val_fist) to the root image directory.

- Complete Source Path: The source_img_path is formed by combining the folder location with the specific image_id and a .jpg extension.

- Destination Mapping: Two target paths are generated for the final dataset: dest_img_path for the image file and dest_label_path for the upcoming YOLO text label.

Physical File Transfer

- Copying: If verified, shutil.copyfile() performs a direct copy of the image from the HaGRID source directory to the newly established YOLO folder structure.

4. Format the annotation and create the txt file

This stage converts image metadata into the specific text format required by the YOLO architecture.

with open(dest_label_path, 'w') as f:

for i, bbox in enumerate(bboxes):

# Original format: [x_min, y_min, w, h] (normalized)

x_min, y_min, w, h = bbox

# Format for YOLO requires class_id and normalized bbox where [x_center, y_center, w, h]

# We start by getting the label for THIS specific bbox

label_for_bbox = row['labels'][i]

#since we have other classes now, we want to map anything not in class_map to 'others'

if label_for_bbox not in class_map: # if label not in our defined classes (fist or palm) just place it in others

# this will make sure all classes that arent fist or palm go to 'others'

label_for_bbox = 'others'

# convert label to class_id (0: fist, 1: palm, 2: others)

class_id = class_map[label_for_bbox]

# Convert to YOLO format [x_center, y_center, w, h]

x_center = x_min + (w / 2)

y_center = y_min + (h / 2)

# Write the line to the label file

f.write(f"{class_id} {x_center} {y_center} {w} {h}\n")

Label Mapping and Class Assignment

- File Creation: A new text file is opened at the dest_label_path with write permissions ('w').

- BBox Enumeration: The system iterates through every bounding box (bbox) associated with an image.

- Class Logic: For each box, the code retrieves its label and checks it against the defined categories.

- Class Consolidation: If a label is not "fist" or "palm," it is automatically reassigned to the "others" category.

- ID Conversion: The final label is converted into a numeric class_id using the pre-defined class_map (e.g., fist = 0, palm = 1, others = 2).

Mathematical Coordinate Transformation

The system transforms the bounding box from the HaGRID format to the YOLO standard:

- Source Format (HaGRID): xmin,ymin,w,h.

- Target Format (YOLO): xcenter,ycenter,w,h.

- Calculation:

  - xcenter=xmin+w2.

  - ycenter=ymin+h2.

Output Generation

Each processed box is written as a single line in the .txt file following the standard YOLO convention: <class_id> <x_center> <y_center> <w> <h>. This ensures the model receives a standardized input regardless of the original dataset's formatting.

## 5.5 Model Training Configuration

The training phase initializes the YOLOv8n model using the prepared dataset and specific hyperparameters to optimize gesture recognition.

yaml_path = './yolo_dataset/dataset.yaml'

results_dir = './Final_Hand_Gesture_Training'

model = YOLO('yolov8n.pt')  # Load a pre-trained YOLOv8n model

results = model.train(

data=yaml_path,

epochs=30,

imgsz=384,

project=results_dir,

name='HG_final_run',

batch= -1,

workers= 4           # <<<

)

print("\n--- Training Complete! ---")

Key Training Parameters

- Pre-trained Weights (yolov8n.pt): The model starts with knowledge from a massive dataset (COCO), which is then fine-tuned for hand gestures to improve accuracy and save time.

- Epochs (30): The model iterates through the entire training set 30 times to learn patterns and features.

- Image Size (imgsz=384): All images are resized to $384 \times 384$ pixels. This matches the HaGRID 384p dataset specifications and ensures the model processes data efficiently.

- Batch Size (-1): Setting the batch size to -1 enables Auto-Batch mode, which automatically determines the maximum batch size your GPU memory can handle.

- Workers (4): Utilizing 4 workers enables parallel data loading, allowing the CPU to prepare images in the background while the GPU performs the training calculations.

## 5.6 Results of Training

1. P (Precision) = 0.987: Out of all the bounding boxes the model PREDICTED, {98.9%} were correct.

2. R (Recall) = 0.989: Out of all the objects that ACTUALLY EXISTED, the model successfully found of them.

Intersection Over Union (IoU)

The IoU determines how well the model's predicted bounding box overlaps with the Ground Truth (actual) bounding box. Basically how close the predicted box IS TO THE ground truth bounding box

IoU=Area of OverlapArea of Union

Rule: For a detection to even be considered a True Positive, its IoU must be greater than or equal to a set IoU Threshold (e.g., 0.50).

Average Precision (AP): Average Precision is calculated per class (e.g., the AP for 'fist' or the AP for 'palm').

AP is the area under the Precision-Recall curve.

This curve plots Precision against Recall at various confidence thresholds. It captures the trade-off: when the model is very confident (high threshold), Precision is high but Recall is low. When the model is less confident (low threshold), Recall is high but Precision drops.

The higher the AP, the better the model performs for that specific class.

Mean Average Precision (mAP): The "Mean" in mAP means you average the AP scores across all classes in your dataset.

mAP=1Nclassesi=1NclassesAPi

This provides a single number that summarizes the overall quality of the model across all categories.

3. Mean Average Precision (mAP50) = 0.994: model achieved 99.4% accuracy when requiring only a 50% overlap with the true box.

4. map50-95 = 0.871: The average precision calculated across a range of overlap thresholds from 50% to 95%.

# 6. Testing

After the training phase, the model is evaluated using the test split, a set of images that the model has never encountered before. This step confirms how well the system performs in real-world scenarios.

test_model = YOLO('/content/projectcv/Final_Hand_Gesture_Training/HG_final_run/weights/best.pt')

# 2. Run validation on the 'test' split defined in your YAML

metrics = test_model.val(split='test')

# 3. Print the main results

print(f"Test Precision: {metrics.results_dict['metrics/precision(B)']:.3f}")

print(f"Test Recall:    {metrics.results_dict['metrics/recall(B)']:.3f}")

print(f"Test mAP50:     {metrics.results_dict['metrics/mAP50(B)']:.3f}")

Evaluation Steps

- Weight Selection: The system loads best.pt, which contains the specific parameters that achieved the highest accuracy during the 30-epoch training run.

- Test Split Validation: The val(split='test') command directs the model to analyze the 10% of data reserved specifically for final testing.

- Result Extraction: Key performance data is pulled from the metrics object to verify the model's reliability.

Final Test Results

The model demonstrated high stability and accuracy on the unseen test data:

- Test Precision (0.975): 97.5% of the gestures detected by the model were correct.

- Test Recall (0.973): The model successfully identified 97.3% of all actual gestures present in the test images.

- Test mAP50 (0.990): The model maintained a 99% accuracy rate when using the standard 50% overlap threshold.

# 7. Preprocessing

To maximize detection accuracy, the project implements a Two-Pass Pipeline architecture. This method focuses the model's computational "attention" on the hand by removing background noise before the final classification.

1) The Two-Pass Logic

- First Pass (Localization): The model scans the raw camera frame to identify the presence of a hand and generates bounding box coordinates (x1, y1, x2, y2).

- Preprocessing Stage: The localized coordinates are passed into a custom function to isolate the hand from the background.

- Second Pass (Classification): The model runs a second time on the "cleaned" image to determine if the gesture is a Fist, Palm, or Others.

def preprocess(frame, box):

x1, y1, x2, y2 = box

# Blur the entire frame

blurred = cv2.GaussianBlur(frame, (41, 41), 0)

# Copy the sharp hand region back from original frame

blurred[y1:y2, x1:x2] = frame[y1:y2, x1:x2]

return blurred

1. Global Gaussian Blur: The entire frame is processed with a (41, 41) kernel, effectively neutralizing background distractions like furniture, people, or complex textures.

2. ROI Restoration: Using the coordinates from the first pass, the original sharp pixels of the hand (the Region of Interest) are "pasted" back onto the blurred image.

3. Result: The model is presented with a perfectly sharp hand against a soft, featureless background, significantly reducing the chance of misclassification.

# 8. Real-Time Inference and Interaction Loop

The final component of the AI system is the execution loop, which integrates the trained model with a live camera feed to perform real-time gesture recognition.

## Core Implementation Logic

The system uses a continuous loop to capture video frames and apply the Two-Pass Pipeline for high-accuracy detection:

model = YOLO(os.path.join(results_dir, 'HG_final_run', 'weights', 'best.pt'))

cap = cv2.VideoCapture(0) # 0 is the default camera

while True:

ret, frame = cap.read()

firstpass1 = model(frame, verbose =False)

fpannotated = firstpass1[0].boxes # Get the boxes

Camera Initialization: The cv2.VideoCapture(0) command opens the default system camera to begin the data stream.

Coordinate Extraction: The model outputs bounding boxes in a GPU-friendly format, which are extracted inside the variable fpannotated

if len(fpannotated) > 0: # If there are any boxes detected

for box in fpannotated:

x1, y1, x2, y2 = box.xyxy[0].cpu().numpy().astype(int)

preprocessed_roi = preprocess(frame, (x1, y1, x2, y2))

secondpass = model(preprocessed_roi, verbose =False)

Coordinate Conversion: The model outputs bounding box data in a GPU-friendly tensor format. The system uses .cpu().numpy().astype(int) to move this data to the CPU and convert it into standard integers representing pixel positions xmin,ymin,xmax,ymax.

Selective Preprocessing: The extracted coordinates are passed to the preprocess function, which blurs the background while keeping the specific hand region sharp.

Pipeline: The secondpass model then analyzes the "cleaned" frame. This ensures that the final gesture prediction (Fist or Palm) is based on data with minimal background interference.

if len(secondpass[0].boxes) > 0:

# Get the first (highest confidence) detection

gesture_box = secondpass[0].boxes[0]

# Get the Class ID (number) and convert to Class Name (string)

cls_id = int(gesture_box.cls[0]) # Class ID as integer

class_name = model.names[cls_id] # Class name from model's names dictionary

conf = float(gesture_box.conf[0]) # Confidence score as float

Top Result Selection: By accessing boxes[0], the system focuses only on the most probable detection, ensuring the dustbin responds to the clearest command.

Class Mapping: The numeric cls_id (0, 1, or 2) is mapped back to its string name (fist, palm, or others) using the model.names dictionary established in the YAML configuration.

Confidence Quantification: The conf value provides a percentage (0.0 to 1.0) of how certain the model is about the detected gesture.

## Drawing Bounding Boxes for feedback

if class_name != 'others':

# 1. Create the annotated image (Draw the box)

spannotated = secondpass[0].plot()

label_text = f"{class_name} {conf:.2f}"

(w, h), _ = cv2.getTextSize(label_text, cv2.FONT_HERSHEY_SIMPLEX, 0.6, 2)

# Draw a filled rectangle behind the text so it's readable

if class_name == 'fist':

cv2.rectangle(frame, (x1, y1), (x2, y2), (0, 255, 0), 2) # Draw the bounding box

cv2.rectangle(frame, (x1, y1), (x1, y1), (0, 255, 0), -1)

if class_name == 'palm':

cv2.rectangle(frame, (x1, y1), (x2, y2), (255, 0, 0), 2) # Draw the bounding box

cv2.rectangle(frame, (x1, y1), (x1, y1), (255, 0, 0), -1) # write the label

# Draw the text on the SHARP 'frame'

# We use (x1, y1 - 5) to put it just above the hand box

cv2.putText(frame, label_text, (x1, y1 - 5),

cv2.FONT_HERSHEY_SIMPLEX, 0.6, (255, 255, 255), 2)

To maintain a clean user interface, the system implements a conditional filter:

- Class Exclusion: The code specifically checks if the detected class_name is not 'others', ensuring only actionable commands trigger visual feedback.

- Dynamic Labeling: A string containing the gesture name and its confidence score (e.g., "fist 0.98") is prepared for the display.

Color-Coded Feedback

The system applies specific color logic to distinguish between the two primary commands:

- Fist (Come to User): A green bounding box (BGR: 0, 255, 0) is drawn around the hand to signify the "forward" command.

- Palm (Go Back): A blue bounding box (BGR: 255, 0, 0) is drawn to signify the "return" command.

- Text Overlay: Using cv2.putText, the gesture name is rendered just above the bounding box coordinates (x1, y1 - 5) in white for maximum readability.

# 9. Bonus: Hardware Interface to Robot

Takes input from camera, makes a prediction and sends a command to the esp-32 cam over Wi-Fi based on the prediction. The command drives motor output, until another command is received.

## 9.1 Laptop Code

Get’s model prediction, converts it to command, and sends it over wifi to the esp-32 cam.

import socket

import time

#########################

# Network Configuration

#########################

MICROCONTROLLER_IP = "192.168.1.50"  # set to ESP32-CAM IP

PORT = 5005

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

#########################

# Gesture Mapping

#

# Change keys to match YOUR model's class names.

#########################

GESTURE_MAP = {

"palm": "forward",

"fist": "stop",

}

#########################

# Load YOLOv8 Model

#########################

# Replace path with your model

model = YOLO("./Hand_Gesture_Training/HG_final_run/weights")

#########################

# Video Capture Device

#########################

cap = cv2.VideoCapture(0)  # Adjust if needed

def get_gesture_output():

"""

Captures an image, runs YOLOv8 inference, and returns:

'forward', 'backward', or None

based on the highest-confidence gesture detection.

"""

ret, frame = cap.read()

if not ret:

return None

# Inference

results = model(frame, verbose=False)[0]

if len(results.boxes) == 0:

return None

# Find highest confidence detection

boxes = results.boxes

best = boxes[boxes.conf.argmax()]

class_id = int(best.cls)

class_name = model.model.names[class_id]

# Convert gesture to command via lookup

return GESTURE_MAP.get(class_name, None)

def send_command(command: str):

if command not in ("forward", "backward"):

return

sock.sendto(command.encode("utf-8"), (MICROCONTROLLER_IP, PORT))

print(f"Sent: {command}")

def main_loop():

last_command = None

while True:

gesture_action = get_gesture_output()

if gesture_action and gesture_action != last_command:

send_command(gesture_action)

last_command = gesture_action

time.sleep(0.1)

if __name__ == "__main__":

try:

main_loop()

except KeyboardInterrupt:

cap.release()

print("Shutting down.")

## 9.2: Microcontroller Code

Receives command, and drives motors

#include <WiFi.h>

#include <WiFiUdp.h>

const char* ssid     = "YOUR_WIFI_SSID";

const char* password = "YOUR_WIFI_PASSWORD";

WiFiUDP udp;

const int UDP_PORT = 5005;

// L298N motor driver control pins

const int A_IN1 = 12;

const int A_IN2 = 13;

const int B_IN1 = 15;

const int B_IN2 = 14;

void setup() {

Serial.begin(115200);

pinMode(A_IN1, OUTPUT);

pinMode(A_IN2, OUTPUT);

// Stop motors initially

digitalWrite(A_IN1, LOW);

digitalWrite(A_IN2, LOW);

pinMode(B_IN1, OUTPUT);

pinMode(B_IN2, OUTPUT);

// Stop motors initially

digitalWrite(B_IN1, LOW);

digitalWrite(B_IN2, LOW);

WiFi.begin(ssid, password);

Serial.print("Connecting");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nConnected to WiFi");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

udp.begin(UDP_PORT);

Serial.printf("Listening on UDP port %d\n", UDP_PORT);

}

void driveForward() {

digitalWrite(A_IN1, HIGH);

digitalWrite(A_IN2, LOW);

digitalWrite(B_IN1, HIGH);

digitalWrite(B_IN2, LOW);

}

void driveBackward() {

digitalWrite(A_IN1, LOW);

digitalWrite(A_IN2, HIGH);

digitalWrite(B_IN1, LOW);

digitalWrite(B_IN2, HIGH);

}

void stopMotors(){

digitalWrite(A_IN1, HIGH);

digitalWrite(A_IN2, HIGH);

digitalWrite(B_IN1, HIGH);

digitalWrite(B_IN2, HIGH);

}

void loop() {

int packetSize = udp.parsePacket();

if (packetSize) {

char buffer[32];

int len = udp.read(buffer, sizeof(buffer) - 1);

if (len > 0) buffer[len] = 0;

// FIXED

String command = String(buffer);

command.trim();

if (command == 0) {

driveForward();

Serial.println("Forward command received.");

}

else if (command == 1) {

driveBackward();

Serial.println("Backward command received.");

}

else if(command == 2){

stopMotors();

Serial.println("Stop command received.");

}

}

}

# Conclusion

To conclude, this project trains an AI computer vision model to perform gesture recognition and based on the predicted output, it sends a command to the ESP-32 to control a motorized object.

The Hand-Gestured Dust Bin project represents a successful fusion of modern computer vision and low-cost microcontrollers. By achieving a 98.9% Precision and an 0.8ms Inference Speed, the system proves that real-time, gesture-based control is not only possible but highly reliable for everyday tasks.

Through the implementation of the double-pass processing pipeline, we have created a model that is resilient to environmental noise and background clutter. This project serves as a foundational step toward a world where technology adapts to human behavior, providing intuitive and touchless solutions for a more accessible future.